The bearing capacity of a pavement can be measured in situ and in an indirect way. For this goal often deflection measurements are used on public roads: the commonly used  Benkelman beam, the Lacroix deflectograph and the nowadays developed Falling Weight Deflectometer (FWD).The FWDs they are capable measurement of the vertical deflections from the load in optional distance. It is possible to fit a function onto the discreet deflection measuerement values. The fitted function offers a fast definable methods for the radius of curvature calculation. Importance of the radius of curveture in the estimitation of the bearing capacity of the pavement was early well knowd.The remaining lifetime of structure layers is a very important question for the civil engineers. In the course of the calculation we found that the the critical extension directly based on the radius of curvature.

Falling Weight Deflectometer (FWD) are widely used for deflection measurements of road pavements. The FWDs can measure the vertical deflections of the pavement in several positions, which distance from the load are optional. In this paper we introduce a mathematical function, which can be fitted to the discreet deflection measurement values. The fitted function offers several analysis  methods,  for  example  the  calculation  of  the  radius  of curvature. Importance of the radius of curveture in the estimitation of the bearing capacity of the pavement is well known. Knowing the maximal curvature we can calculate the extension of the asphalt layer, which strongly correlates to the remaining lifetime of the layer.

The paper introduces a new method for calculating the elastic moduli of the pavement layers. The method requires only two input parameters: the thickness of the upper „bound” layer and the falling weight deflectometer (FWD) measurement data. The author developed a continuously differentiable regression function, which can be applied to describe the shape of the deflection bowl. Additional parameters of the deflection bowl (e.g. radius of curvature, position of the inflexion point) can be calculated based on the regression function. FWD measurements were simulated running the „BISAR” software on different pavement variations. Outputs of the simulations were further processed with own developed software. As a result, a series of diagrams were elaborated, by which the elastic moduli of the pavement layers can be determined.